# Chuyển mạch công nghiệp POE 1000Mb

# MKISP12-5510

## Thông tin sản phẩm

- MKISP12-5510 là switch công nghiệp Gigabit Ethernet PoE quản lý được

- Hỗ trợ chuẩn PoE IEEE 802.3bt/at/af

- Công suất cấp nguồn lên đến 90W mỗi cổng

- Truyền dữ liệu và cấp nguồn qua cáp Ethernet

- Hỗ trợ thiết bị PoE PD chuẩn IEEE 802.3bt/at/af

- Tích hợp quản lý nguồn PoE và tối ưu công suất

- Giảm nhu cầu đi dây nguồn riêng cho thiết bị đầu cuối

## Tính năng chính

- 8 cổng PoE 10/100/1000BASE-TX và 4 cổng uplink quang 10G SFP+

- Hỗ trợ PoE chuẩn IEEE 802.3bt/at/af

- Hỗ trợ ERPS Ring phục hồi <20ms

- Thiết kế công nghiệp DIN-Rail vỏ kim loại chuẩn IP50

- Hỗ trợ nguồn DC dự phòng 48V~57V

- Quản lý qua CLI/Web/SNMP

- Hỗ trợ DI 4 chân (2 Digital Input), lắp DIN-Rail hoặc treo tường

- Hỗ trợ TIP OpenLAN Switching (OLS), tích hợp uCentral và quản lý qua OpenLAN cloudController

- Chống sét lan truyền 6KV cho cổng RJ-45 downlink

| Product Model |  | MKISP12-5510 |
| --- | --- | --- |
| Product Image |  |  |
| Port | RJ-45 10/100/1000BASE-T Ports | 8 |
|  | SFP+ 10 Gigabit Uplink Ports (1000M/10G auto sensing) | 4 |
|  | RJ-45 Console Port | 1 |
| Performance | Switching Capacity | 96 Gbps |
|  | MTBF | Mean Time Between Failure > 1,000,000 hours@25 ˚C |
|  | Flash Memory | 64 MB |
|  | DDR3 | 512 MB |
|  | Packet Buffer | 12 Mb |
|  | MAC Address Table Size | 16 K |
|  | Jumbo Frames | 12 KB |
|  | Auto-negotiation, Auto-MDI/MDIX | Yes |
| Mechanical | Installation Option | DIN-rail and wall mount |
|  | Dimension (W x D x H) | Est. 93mm x 127mm x 134mm (W x D x H) |
|  | Weight | 1.4 kg |
| PoE | IEEE 802.3bt/802.3at/802.3af | Yes |
|  | Ultra PoE | Yes |
|  | PoE Power Budget | 480 W |
|  | PoE Scheduling | Yes |
| Power Supply | Support redundant 48V~57V DC power input | Yes |
| Environmental | Operating Temperature | -40 ~ 75 ˚C |
|  | Storage Temperature | -40 ~ 85 ˚C |
|  | Humidity (non-condensing) | 10 ~ 95% (non-condensing) |
|  | Environmental Regulation Compliance: WEEE | Yes |
|  | Environmental Regulation Compliance: RoHS | Yes |
| Certification | FCC class A | Yes |
|  | CE | Yes |
|  | Safety Compliance: CB | Yes |
|  | Safety Compliance: UL | Yes |

### L2 Features

- Switch Ports:

8 x 10/100/1000BASE-TX PoE ports

FP+ 10 Gigabit Uplink Ports (1000M/10G auto sensing)

1 x RJ-45 Console Port 1 x Reset Button

4-pin DI terminal (2 x Digital Input)

- Flow Control:

IEEE 802.3x for full-duplex mode Back-Pressure for half-duplex mode

- Jumbo frames 12KB

- Broadcast/multicast/unknown unicast storm control

- Spanning Tree Protocol:

IEEE 802.1D Spanning Tree Protocol (STP)

IEEE 802.1w Rapid Spanning Tree Protocol (RSTP)

IEEE 802.1s Multiple Spanning Tree Protocol (MSTP), 64 instances BPDU Guard

BPDU filtering Root Guard

BPDU transparent Loopback detection

- Non-Spanning Tree Loopback detection

- ITU-T G.8032 Ethernet Ring Protection: Sub 50 msec convergence

Revertive operation mode Multiple-ring network

- VLANs:

Supports 4K VLAN Port-based VLAN IEEE 802.1Q VLAN GVRP

IEEE 802.1v protocol-based VLAN IP subnet-based VLAN

MAC-based VLAN Traffic Segmentation

- L2 Virtual Private VLAN: Q-in-Q

L2 protocol tunneling (xSTP, CDP, VTP and PVST+, LLDP) CDP/PVST+ filtering

Selective Q-in-Q

- Link Aggregation:

Static Trunk

IEEE 802.3ad Link Aggregation Control Protocol Trunk Groups: 16, up to 8 GE/4 10G ports per group Load Balancing: SA+DA, SA, DA, SIP+DIP, SIP, DIP

- IGMP Snooping:

IGMP v1/v2/v3 Snooping IGMP Proxy reporting IGMP Filtering

IGMP Throttling

IGMP Immediate Leave IGMP Querier

- MVR (Multicast VLAN Registration): Supports 5 multicast VLANs

- Port mirroring

- Remote port mirror (RSPAN)

### QoS Features

- Priority Queues: 8 hardware queues per port

- Traffic Classification: IEEE 802.1p CoS

IP Precedence DSCP

MAC access control list (source/destination MAC, Ether type, priority ID/VLAN ID)

IP standard access control list (source IP)

IP extended access control list (source/destination IP, protocol, TCP/UDP port number)

- Traffic Scheduling: Strict Priority Weighted Round Robin Strict +WRR

- Ingress policy map (police rate, remark CoS)

- Egress policy map (police rate, remark CoS/DSCP)

- Rate Limiting (ingress and egress, per port base) GE: Resolution 64 Kbps ~1,000 Mbps

10G: Resolution 64 Kbps ~10,000 Mbps

- Auto Traffic Control

### Security

- Port security

- IEEE 802.1X port-based and MAC-based authentication

- Dynamic VLAN Assignment, AutoQoS

- MAC authentication

- Web authentication

- Voice VLAN

- Guest VLAN

- L2/L3/L4 access control list:

MAC access control list (source/destination MAC, Ether type, priority ID/VLAN ID)

IP standard access control list (source IP)

IP extended access control list (source/destination IP, protocol, TCP/UDP port number)

- IPv6 ACL

- DHCP Snooping

- DHCP Option 82

- DHCP Option 82 Relay

- IP Source Guard

- PPPoE IA

- Dynamic ARP Inspection

- Denial of Service

- Login security

- RADIUS authentication

- RADIUS accounting

- TACACS +authentication

- TACACS +accounting

- TACACS +authorization

- Management interface sccess filtering (SNMP, Web, Telnet)

- SSH (v2) for secure Telnet

- SSL for HTTPS

- SNMP v1/v2c/v3

### Green Ethernet

- IEEE 802.3az Energy-Efficient Ethernet (EEE)

### IPv6 Features

  - IPv4/IPv6 dual protocol stack

  - IPv6 Address Types Stack: Unicast

  - IPv6 Neighbor Discovery: Duplicate address Address resolution

Unreachable neighbor detection

  - Stateless auto-configuration

  - Manual configuration

  - Remote IPv6 ping

  - IPv6 Telnet support

  - HTTP over IPv6

  - SNMP over IPv6

  - IPv6 Syslog support

  - IPv6 TFTP support

  - MLD Snooping v1/v2

  - IPv6 source guard

  - DHCPv6 snooping

  - MVR6

### Management

  - Switch Management:

CLI via console port or Telnet/SSH Web management

SNMP v1, v2c,v3

  - Firmware and Configuration:

Firmware upgrade via TFTP/HTTP/FTP/SFTPserver Multiple configuration files

Configuration file upload/download viaTFTP/HTTP/FTP/SFTP

server

  - RMON (groups 1, 2, 3 and 9)

  - BOOTP, DHCP client for IP address assignment

  - DHCP dynamic provision option 66,67

  - SNTP

  - Syslog (local flash)

  - Remotelog (RFC3164)

  - SMTP (email notification)

  - Support LLDP (802.1ab)

  - sFlow v4,v5

### Routing

  - IPv4/IPv6 static route

  - RIP v1/v2

### Dying Gasp

6KV Surge Protection (BASE-T Ports) Power Supply

  - Power Input Voltage: 48V~57V DC Connector: 6pin removable terminal for DC Power in & Relay out

### Environmental Specifications

  - Emission/Safety/Shock/Freefall/Vibration CE/FCC class A

EN61000-4-2 EN61000-4-3 EN61000-4-4 EN61000-4-5 EN61000-4-6 EN61000-4-8

Safety:

EN IEC 62368-1:2020+A11:2020

Shock:

IEC60068-2-27

Freefall:

IEC60068-2-31

Vibration:

IEC60068-2-6

### Environmental Specifications

  - Temperature:

-40˚C to 75˚C standard operation

-40˚C to 85˚C (Storage)

  - Humidity: 10% to 95 % (non-condensing)

Warranty

Please check https://mk.com.vn/ for the warranty terms in your

country.

About MK Group Networks Corporation

MK Group Networks Corporation is in the business of providing innovative network solutions. In the service provider network, in the data center or in the cloud, MK Group Networks Corporation delivers the soﬅware and systems that transform the way the world connects. MK Group Networks Corporation serves customers and partners worldwide. Additional information can be found at www.edge-core.com.

MK Group Networks Corporation is a subsidiary of Accton Technology Corporation, the leading network ODM company. The MK Group data center switches are developed and manufactured by Accton.

To purchase MK Group Networks solutions, Tầng 2 Tòa nhà The Vista, số 4 ngõ 15 Duy Tân, Cầu Giấy, Hà Nội, Việt Nam Điện thoại : +84-24 7100 6781

© Copyright 2025 MK Group Networks Corporation. The information contained herein is subject to change without notice. This document is for informational purposes only and does not set forth any warranty, expressed or implied, concerning any equipment, equipment feature, or service ofiered by MK Group Networks Corporation. MK Group Networks Corporation shall not be liable for technical or editorial errors or